Code of practice for Design and Performance of Remote Monitoring & Diagnostics Solution for Lifts

远程监控和诊断解决方案设计和性能的实施规范

This code of practice is electronically published by the Building and Construction Authority

1

Copyright Notice and Disclaimer:

版权声明和免责声明：

All material in this document are protected by copyright, trademark, and other forms of proprietary rights, and are the property of the Building Construction Authority (“BCA”). You are requested to refrain from copying, reproducing, uploading, distributing, publishing, posting, modifying, creating derivative works, transmitting or in any way exploiting any such content. The information in this document is subject to change and shall not be treated as constituting any advice to any person. If you wish to use the content for any other purpose, please seek prior consent from BCA. BCA shall not be responsible or liable for any consequences (financial or otherwise) or any damage or loss suffered, directly or indirectly, by any person resulting or arising from the use of or reliance on any information in this document.

本文件中的所有材料均受版权、商标和其他形式的专有权保护，是建筑与建设局（"BCA"）的财产。请您不要复制、复印、上传、分发、发布、张贴、修改、创建衍生作品、传输或以任何方式利用此类内容。本文件中的信息可能会发生变化，不应被视为对任何人构成任何建议。如果您希望将内容用于其他目的，请事先征得BCA的同意。BCA对因使用或依赖本文件中的任何信息而导致或引起的任何后果（包括财务或其他方面的后果）、任何损害或损失，不承担责任。

2

Contents目录

| Preface .前言............................................................................................................................... |  |  | 4 |
| --- | --- | --- | --- |
| 1. | Introduction 介绍................................................................................................ |  | 5 |
| 2. | Definitions and terminology 定义和术语.......................................................... |  | 5 |
| 3. | Remote Monitoring & Diagnostics 远程监控和诊断................................................... |  | 7 |
| 3.1 |  | General requirements 一般要求.......................................................................................... | 7 |
| 3.2 |  | System architecture 系统架构......................................................................................... | 7 |
| 3.3 |  | Data Acquisition数据采集..................................................................................................... | 7 |
| 3.3.1 |  | General Requirements 一般要求..................................................................................... | 7 |
| 3.3.2Intrusive or Non-intrusive Data Acquisition System侵入式或非侵入式数据采集系统. |  |  | 7 |
| 3.4 |  | Data Pre-Processing数据预处理....................................................................................... | 8 |
| 3.5 |  | Transmission and Storage of Data 数据传输和存储....................................................... | 8 |
| 3.6 |  | Data Analytics数据分析........................................................................................................ | 8 |
| 3.7 |  | Use of the Data Analytics数据分析的应用......................................................................... | 8 |
| 3.7.1 |  | General Requirements一般要求........................................................................... | 8 |
| 3.7.2 |  | Monitoring Outcomes 监测结果...................................................................................... | 8 |
| 3.7.3 |  | Feedback System反馈系统............................................................................................. | 9 |
| 4. | Data Visualisation数据可视化................................................................................................. |  | 9 |
| 5. | Remote Testing, Intervention and Control远程测试、干预和控制................................. |  | 10 |
| 6. | Cybersecurity Requirements 网络安全要求. |  | 11 |
| 6.1 |  | Scope范围............................................................................................................................. | 11 |
| 6.2 |  | General Requirements 一般要求............................................................................... | 11 |
| 6.3 |  | Architecture and Design架构和设计............................................................................... | 11 |
| 6.4 |  | Product Development Lifecycle产品开发生命周期............................................. | 11 |
| 6.5 |  | Component and System Security组件和系统安全............................................. | 11 |
| Annex A附录 A................................................................................................................................. |  |  | 12 |
| Annex B附录 B......................................................................................................................... |  |  | 14 |
| Acknowledgements致谢...................................................................................................... |  |  | 15 |

3

Preface 前言

- With Singapore becoming an increasingly built-up nation, there will be a greater reliance on lifts. As the number of lifts increases, the industry faces challenges in raising its workforce capacity and capabilities to meet the increased demand for maintenance services as well as evolving public expectations. Advancements in technology have presented opportunities for the lift industry to achieve better productivity and move towards a leaner workforce. This can be achieved through the adoption of technology such as Remote Monitoring & Diagnostics that would enable existing work processes to be re-designed and streamlined, thereby enhancing manpower efficiency and achieving better lift performance.

随着新加坡不断成为一个越来越建设完善的国家，人们对电梯的依赖程度也越来越高。随着电梯数量的增加，电梯行业面临着提高劳动力能力和能力以满足维护服务的增加需求以及不断演变的公众期望的挑战。技术的进步为电梯行业提供了机会，实现更好的生产力，并向更精简的劳动力发展。通过采用远程监控和诊断等技术，现有的工作流程可以得到重新设计和优化，从而提高人力资源效率并实现更好的电梯性能。

- The aim of this code is to set the fundamental requirements and guidelines for the use of Remote Monitoring & Diagnostics for lifts in Singapore.

本准则的目标是为新加坡电梯远程监控和诊断的使用设立基本要求和指南。

- The Building and Construction Authority would like to extend its sincere appreciation to the lift industry and other partners for contributing invaluable effort, time and inputs towards the development of this code of practice.

新加坡建筑与建设管理局衷心感谢电梯行业和其他合作伙伴为制定本操作准则所做出的宝贵努力、时间和贡献。

4

1.  Introduction 介绍

- Scope 范围

    - This code shall apply to the design and performance of Remote Monitoring & Diagnostics Solution for lifts in Singapore.

本准则适用于新加坡电梯远程监控和诊断解决方案的设计和性能。

- Purpose 目的

1.2.1 The purpose of this code is to establish minimum standards for lifts connected to Remote Monitoring & Diagnostics Solution in the Republic of Singapore. The code is written with the intent to inform lift owners and contractors about the baseline requirements for an acceptable Remote Monitoring & Diagnostics Solution in Singapore.

本准则的目的是为连接到新加坡远程监控和诊断解决方案的电梯建立最低标准。本准则的编写意在向电梯业主和承包商传达新加坡可接受的远程监控和诊断解决方案的基本要求。

2.  Definitions and terminology定义和术语

For the purpose of this code, the following definitions apply:

针对本准则，适用以下定义

- Big Data大数据

Big Data refers to extremely large data sets collected from the lifts - both structured or unstructured that can be analysed to provide insights on the performance of the lifts.

大数据是指从电梯中收集的极其庞大的数据集，包括结构化或非结构化数据，可以进行分析以提供有关电梯性能的洞察。

- Big Data analytics大数据分析

Big Data analytics, an advanced data analysis tool that applies data mining, predictive analytics and machine learning to sets of Big Data.

大数据分析是一种高级的数据分析工具，它应用数据挖掘、预测分析和机器学习技术对大数据集进行分析。

- Data Mining数据挖掘

Data Mining, a form of data analytics that involves sorting through large data sets to identify trends, patterns and relationships.

数据挖掘是一种数据分析的形式，涉及对大型数据集进行排序，以识别趋势、模式和关系。

- Data Visualisation数据可视化

Data Visualisation, refers to the use of statistical graphics, plots, information graphics and other form of tools to communicate information in a clear and efficient manner

数据可视化是指使用统计图形、绘图、信息图形和其他工具来以清晰高效的方式传达信息。通过数据可视化，可以将复杂的数据以图形化的方式呈现，帮助人们更容易地理解和分析数据。数据可视化的目的是通过视觉化的方式展示数据，使用户能够迅速理解数据的含义和趋势，并支持决策过程。数据可视化提供了一种直观的方式来展示数据，使得观察者能够快速地获取信息并发现潜在的见解。

- Discrete Signals离散信号

Discrete Signals are signals that can report on two states, such as either on or off, open/closed.

离散信号是指只能报告两种状态的信号，比如开或关、打开或关闭等。它们具有明确的状态，并且不会存在中间状态。离散信号通常用于表示二元逻辑或控制系统中的状态，其中只存在两种可能的取值。例如，在开关控制中，离散信号可以表示开关的状态，取值为开或关；在传感器监测中，离散信号可以表示某个事件的发生与否，取值为有或无。离散信号的特点是简单明确，易于处理和理解。

- Edge Analytics边缘分析

Edge Analytics is an approach to data collection and analysis in which an automated analytical computation is performed on-site on data from a sensor, network device or other component instead of waiting for the mostly unprocessed data to be sent back to a centralised data storage for analysis.

边缘分析是一种数据收集和分析的方法，它在现场对传感器、网络设备或其他组件的数据进行自动化分析计算，而不是等待大部分未经处理的数据被发送回集中式数据存储进行分析。

- First Time Fix Rate首次修复率

First Time Fix Rate, indicates the percentage of technical faults which do not re-occur for the next 30 days after the maintenance personnel last resolved the a technical fault.

首次修复率（First Time Fix Rate）指的是在维修人员最后解决技术故障后的30天内，不再出现该技术故障的百分比。

- Hoistway电梯井道

The fixed structure consisting of a shaft way for the travel of one or more lifts.

电梯井道（shaft way）是由固定结构构成的，用于电梯运行的竖直通道。

5

- Internet of Things (IoT) 物联网

Internet of Things, refers to a huge network formed by the combination of a variety of sensors, hardware and etc connected to the internet. The aim is to have all the items connected with the network, and the system can identify the items, locate, track, monitor, and trigger the corresponding event automatically and in real time.

物联网是指由各种传感器、硬件等组合连接到互联网而形成的庞大网络。其目的是将所有物品与网络连接，使系统能够自动且实时地识别、定位、追踪、监控和触发相应的事件。

- Intrusive Data Acquisition System侵入式数据采集系统

Intrusive Data Acquisition System refers to any means of acquiring data from the lift controller for the purpose of data analysis

侵入式数据采集系统指的是为了数据分析目的而从电梯控制器获取数据的任何手段。

- Machine Learning机器学习

Machine Learning, is an artificial intelligence technique that uses automated algorithms to analyse through various data sets.

机器学习是人工智能领域的一个分支，它利用自动化算法对各种数据集进行分析。

- Non-Intrusive Data Acquisition System非侵入式数据采集系统

Non-Intrusive Data Acquisition System refers to any means of acquiring data, other than from the lift controller, for the purpose of data analysis. Example could be the use of sensors on lift car or inside the hoistway.

非侵入式数据采集系统指的是除了从电梯控制器获取数据之外，为了进行数据分析而采集数据的任何手段。例如，在电梯轿厢或井道内部使用传感器。

- Non-Technical Faults非技术性故障

Non-Technical faults refers to issues in the lifts which have no direct bearing on the safety and reliability of the lifts. Examples are faulty buttons or damaged display indicators.

非技术性故障指的是电梯中与安全性和可靠性没有直接关联的问题。例如，按钮故障或显示指示器损坏等。

- Performance性能

The behaviour of the lift which is determined by one or more measured or calculated parameters such as temperature, vibration, speed or acceleration.

电梯的行为是由一个或多个测量或计算的参数决定的，例如温度、振动、速度或加速度。

- Read-only function只读功能

Read-only function refers to the capability of being accessed but not being changed or deleted.

只读功能指的是能够被访问，但不能被修改或删除的能力。

- Remote Monitoring & Diagnostics (RM&D) Solution远程监控和诊断

Remote Monitoring & Diagnostics refers to the use of technology to automatically track and log the usage, operations, and response of lifts, to detect lift anomalies, predict future anomalies/breakdowns, and provide information for diagnosis and reduction of such anomalies and breakdowns.

远程监控和诊断是指利用技术自动跟踪和记录电梯的使用情况、操作和响应，检测电梯异常，预测未来的异常或故障，并提供信息用于诊断和减少这些异常和故障。

- Rectification Works整改工程

It refers to any activities such as inspection, functional checks, servicing, repairing, or replacing of components with the intent to keep the equipment functioning as originally designed for.

这是指通过检查、功能检测、维修、更换零部件等活动，以确保设备按照最初设计的功能正常运行的意图。

- Sensors传感器

A sensor is a device, module, or subsystem used to detect events or changes in its environment and send the information to a network.

传感器是一种用于检测环境中的事件或变化并将信息发送到网络的设备、模块或子系统。

- Shall应当

Indicates that the requirement is strictly to be followed in order to conform to the standard.

意味着该要求必须严格遵守，以符合标准的规定。

6

- Should应该

Indicates that among several possibilities, one is recommended as particularly suitable, without mentioning or excluding others, or that a certain course of action is preferred but not necessarily required.

表示在多种可能性中，某种选择被推荐为特别适合的，但不排除其他可能性，或者表示某种行动方案是首选但不一定必需的。

- Technical Faults技术故障

Technical faults refer to the malfunction or breakdown of the lifts due to failure of components. External factors such as water ingress, user behaviour-related or natural disasters are not considered as technical failures.

技术故障指电梯由于组件故障而发生的故障或故障。不将水进入、用户行为相关或自然灾害等外部因素视为技术故障。

3.  Remote Monitoring & Diagnostics远程监控和诊断

3.1	General requirements一般要求

- Lifts equipped with Remote Monitoring & Diagnostics (RM&D) shall meet the provisions in the Code.

配备远程监控和诊断（RM&D）系统的电梯应符合本规范的规定。

- The performance of the RM&D should as much as possible, ensure that the maintenance outcomes of the lifts in the prevailing requirements of the Building Maintenance & Strata Management Regulations (Lift, Escalator and Building Maintenance) are tracked and monitored at all times.

RM&D的性能应尽可能确保对电梯的维护结果进行跟踪和监测，以满足《建筑维修和地契管理规例（电梯、自动扶梯和建筑维修）》的现行要求。

- Any electronic devices installed for the purpose of the RM&D Solution shall not compromise the safe operations of the lift.

为了实现RM&D解决方案而安装的任何电子设备不得影响电梯的安全运行。

- The RM&D Solution should support an open standard interface for its performance data, such as Open Platform Communications United Architecture.

RM&D解决方案应支持性能数据的开放标准接口，例如开放平台通信统一架构

3.2	System architecture系统架构

3.2.1 The RM&D Solution system architecture shall consist of the following combinations:

RM&D解决方案的系统架构应包括以下组合：

a) Data Acquisition

b) Data Pre-processing c) Data Analytics

3.3	Data Acquisition数据采集

3.3.1	General Requirements一般要求

- A data acquisition system shall be used to continuously monitor and track the performance data of the lift.

应使用数据采集系统来持续监测和跟踪电梯的性能数据。

- The data acquisition system should be collecting performance data from the lifts when normal power supply to the system is interrupted. In such scenario, it is not necessary to collect any data after the lifts completed their emergency operations and parked at the designated floor.

当系统的正常电源中断时，数据采集系统应收集电梯的性能数据。在这种情况下，不需要在电梯完成紧急操作并停放在指定楼层后继续收集任何数据。

- The data sampling frequency will depend on the performance parameters and the amount of storage available. It should be sufficient such that it does not affect the accuracy and precision of the data analytics.

数据采样频率将取决于性能参数和可用的存储空间。应确保采样频率足够，以不影响数据分析的准确性和精度。

- The data acquisition system shall be of either the intrusive type or non-intrusive type.

数据采集系统应为入侵型或非入侵型之一

3.3.2	Intrusive or Non-intrusive Data Acquisition System侵入式或非侵入式数据采集系统.

3.3.2.1  The intrusive or non-intrusive data acquisition system shall be of the read-only function.

入侵型或非入侵型的数据采集系统应具有只读功能。

7

3.4	Data Pre-Processing数据预处理

- To minimise the latency of data analytics and reduce the strain on the central data diagnostic resources, a data pre-processing unit incorporating edge analytics should be provided.

为了最小化数据分析的延迟并减少对中央数据诊断资源的压力，应提供一个包含边缘分析的数据预处理单元。

- The data pre-processing unit should also be capable of separating performance data collected

from the data acquisition system into different categories. Critical data NOTE should be sent immediately for data diagnostics purposes while less critical data can be uploaded when the network bandwidth is less congested.

数据预处理单元还应能够将从数据采集系统收集到的性能数据分成不同的类别。关键数据应立即发送以进行数据诊断，而较不关键的数据可以在网络带宽较少拥挤时上传。

NOTE Critical data are any information collected from the lift operations that are pertinent in the prediction of imminent breakdowns or safety related issues.

注意：关键数据是指从电梯运行中收集的任何信息，对于预测即将发生的故障或与安全相关的问题具有重要意义

3.5	Transmission and Storage of Data数据传输和存储

3.5.1	The transmission of data shall be through a secure network.

数据传输应通过安全网络进行

3.5.2	Data transmitted shall be stored in physical data centre or IoT server.

传输的数据应存储在物理数据中心或物联网服务器中。

3.5.3	Data transmission and storage should meet security requirements specified in section 6.

数据传输和存储应符合第6节中指定的安全要求。

3.6	Data Analytics数据分析

3.6.1	The analytics tool should be capable of analysing trends, patterns and relationship of data sets in order to identify anomalies in the performance data of the lifts.

分析工具应能够分析数据集的趋势、模式和关系，以识别电梯性能数据中的异常情况。

3.6.2	Data analytics tools such as regression, decision trees, clustering, or machine learning should be employed to predict potential failures of lift components and future breakdowns of the lift.

应采用回归、决策树、聚类或机器学习等数据分析工具，以预测电梯组件的潜在故障和未来的故障。

3.7	Use of the Data Analytics数据分析的应用.

3.7.1 General Requirements一般要求

- The data analytics shall have predictive maintenance capability. It shall be able to issue a recommendation to carry out part replacement, repair or maintenance on the lift before a breakdown occurs. For urgent cases, such as breakdowns or faults which could lead to breakdowns imminently, the system should be able to provide information including the level of urgency for maintenance personnel to rectify the problem immediately. For the less urgent ones, the system should provide information and schedule the maintenance or repair to take place in the coming days or weeks or on the next scheduled maintenance.

数据分析应具备预测性维护能力。它应能在故障发生之前提出建议，对电梯进行部件更换、维修或保养。对于紧急情况，例如即将发生故障或可能导致即将发生故障的故障，系统应能提供包括维修人员立即纠正问题的紧急程度信息。对于不太紧急的情况，系统应提供信息并安排维护或维修在接下来的几天、几周或下次计划的维护中进行。

- The data analytics shall also provide recommendations to improve the maintenance programme, including recommendations and guidance for maintenance personnel to carry out their required maintenance tasks, with the aim of improving the first time fix rate.

数据分析还应提供改进维护计划的建议，包括对维护人员执行所需维护任务的建议和指导，以提高首次修复率为目标。

3.7.2 Monitoring Outcomes监测结果

- The following information corresponding to Table 1 shall be monitored and piped to the data analytics system to generate recommendations on possible rectification works and the suggested timeline to complete them.

与表1对应的以下信息应被监测并传送到数据分析系统，以生成关于可能的纠正工作和建议的完成时间表。

8

|  |  | Table 1: List of Monitoring Outcomes 表1：监控结果列表 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lift system and their sub- |  |  |  | Monitoring Outcomes 监控结果 |  |  |
|  | system电梯系统及其子系统 |  |  |  |  |  |
| 1. Traction Machine |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
|  | 曳引机 |  | identify potential issues with the traction machine; and provide |  |  |  |
|  |  |  | recommendation on possible rectification works and indicate when |  |  |  |
|  |  |  | they are required. 能够监控和分析电梯的性能，并识别牵引机的潜在问题；并提供关于可能的纠正工作的建议，并指示何时需要进行这些工作。 |  |  |  |
| 2. Brakes |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
|  | 制动器 |  | identify potential issues with the brakes; and provide |  |  |  |
|  |  |  | recommendation on possible rectification works and indicate when |  |  |  |
|  |  |  | they are required. 能够监控和分析电梯的性能，并识别制动器的潜在问题；并提供关于可能的纠正工作的建议，并指示何时需要进行这些工作。 |  |  |  |
| 3. Suspension Means |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
|  | 悬挂装置 |  | identify potential issues with the suspension means; and provide |  |  |  |
|  |  |  | recommendations on possible rectification works and indicate when |  |  |  |
|  |  |  | they are required. 能够监控和分析电梯的性能，并识别悬挂装置的潜在问题；并提供关于可能的纠正工作的建议，并指示何时需要进行这些工作。 |  |  |  |
| 4. Guide system (i.e. guide |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
| rail and guide shoes or |  |  | identify potential issues with the guide system; and provide |  |  |  |
| rollers) |  |  | recommendations on the possible rectifications for the guide system |  |  |  |
|  | 导轨系统（包括导轨和导轨鞋或滚轮） |  | and indicate when they are required. 能够监测和分析电梯的性能，识别可能存在的导轨系统问题，并提供对导轨系统的可能纠正措施建议，并指示何时需要进行纠正。 |  |  |  |
| 5. Car and Landing Doors |  |  | To be able to monitor and analyse the performance of lift and |  |  |  |
| (including door protective |  |  | identify potential issues with the car and/or landing doors system; |  |  |  |
| devices) |  |  | and provide recommendations on possible rectification works and |  |  |  |
|  | 轿厢门和层门（包括门保护装置） |  | indicate when they are required. 能够监测和分析电梯的性能，识别可能存在的轿厢和/或门系统问题，并提供可能的纠正工作建议，并指示何时需要进行纠正。 |  |  |  |
| 6. Levelling Devices |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
|  |  |  | identify potential occurrences and instances of mis-leveling; and |  |  |  |
|  |  |  | provide recommendations on possible rectification works and |  |  |  |
|  |  |  | indicate when they are required. 能够监测和分析电梯的性能，识别可能发生的误平层情况，并提供可能的纠正工作建议，并指示何时需要进行纠正。 |  |  |  |
| 7. Fault Diagnosis including |  |  | To be able to monitor and analyse the performance of the lift and |  |  |  |
| the following components: 故障诊断，包括以下组件： |  |  | indicate if one or more of the following fault(s) is/are possible |  |  |  |
| a) | Overspeed Governor |  | cause(s) for the stoppage of the lift: |  |  |  |
| b) Safety Gear |  |  |  | Overspeed Governor Activation |  |  |
| c) | Controller and Inverter |  |  | Safety Gear Activation |  |  |
|  | Drive |  |  Controller and Inverter Drive Failure |  |  |  |
| d) | Buffer |  |  | Buffer Activation |  |  |
| e) Compensation System |  |  |  | Compensation System Activation |  |  |
|  |  |  |  Ascending Car Overspeed Protection Activation |  |  |  |
|  |  |  |  Unintended Car Movement Protection Activation |  |  |  |
|  |  |  |  | Fire Emergency |  |  |
|  | a) 超速器 b) 安全装置 c) 控制器和逆变器驱动器 d) 缓冲器 e) 补偿系统 |  |  | Power Failure |  |  |
|  |  |  |  | 以下是电梯停止运行可能的故障原因，系统将监控并分析电梯的性能，并指示是否发生以下一种或多种故障： - 超速限速器激活 - 安全钳激活 - 控制器和变频器故障 - 缓冲器激活 - 补偿系统激活 - 上行车辆超速保护激活 - 非意图车辆移动保护激活 - 火警紧急情况 - 电源故障 |  |  |

3.7.3 Feedback System反馈系统

- There should be a feedback system to improve the accuracies of the data analytics and allow adaption to change in usage patterns and lift aging.

应该建立一个反馈系统，以提高数据分析的准确性，并允许适应使用模式和电梯老化的变化。

- In addition, there should be a process in place to ensure minimal room for poor or erroneous feedback to the data analytics system. For example, this can be carried out using user-driven confirmation.

此外，应建立一套流程，以确保对数据分析系统的反馈最小程度受到质量低下或错误的影响。例如，可以通过用户驱动的确认来实施此过程。

4.  Data Visualisation数据可视化

- A data visualisation platform shall be provided to the lift owner or his representative by the service provider of the Remote Monitoring & Diagnostics Solution to provide real-time insights on the lift status and performance.

远程监控和诊断解决方案的服务提供商应向电梯业主或其代表提供数据可视化平台，以实时提供有关电梯状态和性能的见解.

9

NOTE Real-time does not imply that data should be displayed without any delay after it is being processed. It is acceptable that the processed data be available after a suitable predetermined period of time.

注意：实时并不意味着数据在处理后立即显示。经过适当的预定时间后，可获得处理后的数据是可以接受的。

- Where a data visualisation platform is provided, it should be a web-based interface and/or a mobile application.

如果提供数据可视化平台，应为基于Web的界面和/或移动应用程序。

- Notifications via mobile applications, text message, emails or other means shall be used to keep the lift owners, or their representatives updated about any abnormal status of the lift at all times.

应使用移动应用程序、短信、电子邮件或其他方式发送通知，以始终将电梯业主或其代表保持对电梯异常状态的最新更新。

- The data visualisation platform should include the following specification of the lift in Table 2 for easy reference:

数据可视化平台应包括以下电梯规格表（表2），以便进行简单参考：

Table 2: List of Recommended Specifications to be shown on the data visualisation platform

表2：在数据可视化平台上显示的推荐规格列表

| S/N | Items | Definition | Denomination |
| --- | --- | --- | --- |
|  | 项目 | 定义 | 电梯名称 |
| 1 | Manufacturer of the | The name of the original | - |
|  | Lift电梯制造商 | equipment manufacturer. 原始设备制造商的名称 |  |
| 2 | Lift Number | Alphanumeric characters for | - |
|  | 电梯编号 | identification of the lift. 用于标识电梯的字母数字字符。 |  |
| 3 | Lift Model | Alphanumeric characters for | - |
|  | 电梯型号 | identification of the lift model. 用于标识电梯型号的字母数字字符。 |  |
| 4 | Lift Type电梯类型 | The primary use of the lift. 电梯的主要用途。 | - |
| 5 | Address of the Lift | The location of the lift in | - |
|  | 电梯地址 | Singapore. 在新加坡的电梯位置 |  |
| 6 | Lift service contractor | The name of the maintenance | - |
|  | 电梯维保承包商。 | contractor. 维护承包商的名称 |  |
| 7 | Year of Installation | The year when the lift was first | YYYY |
|  | 安装年份 | installed. 电梯首次安装的年份。 |  |
| 8 | Number of landings | The number of stops. | - |
|  | served /stops服务楼层数/停站数 | 停靠点数量。 |  |
| 9 | Rated Speed额定速度 | The contract speed of the lift. 电梯的合同速度。 | m/s |
| 10 | Rated Capacity | The maximum load the lift can | kg |
|  | 额定容量 | carry. 电梯可承载的最大负荷 |  |
| 11 | Current Floor Served | An indication on the location of | - |
|  | 当前服务楼层。 | the lift. 电梯所在位置的指示 |  |
| 12 | Current Lift Status | An indication on whether lift is | - |
|  | 当前电梯状态 | moving. 电梯是否在运行的指示 |  |
| 13 | Current Door Status | An indication on whether lift door | - |
|  | 当前门状态 | is opening or closing. 电梯门是打开还是关闭的指示。 |  |

- It should be also possible for users to extract any of the indicator data for analysis (Refer to Annex A for more information):

用户应该能够提取任何指示器数据进行分析（详见附录A获取更多信息）：

  - Indicator data for a particular lift for a specific time period;

特定时间段内特定电梯的指示器数据；

  - Indicator data for a group of lifts in a particular location for a specific time period;

特定时间段内特定位置的一组电梯的指示器数据；

  - Indicator data for a group of lifts maintained by a particular lift service contractor for a specific time period;

特定时间段内由特定电梯服务承包商维护的一组电梯的指示器数据；

  - Indicator data for a group of lifts supplied by an Original Equipment Manufacturer for a specific time period.

特定时间段内由原始设备制造商供应的一组电梯的指示器数据。

5.  Remote Testing, Intervention and Control远程测试、干预和控制

- For Remote Monitoring & Diagnostics Solution that comes with remote testing, control and intervention function, they shall neither interfere with the safety of the lift nor compromise the safety of the users who are using the lifts.

对于具有远程测试、控制和干预功能的远程监控与诊断解决方案，它们既不应干扰电梯的安全，也不应危及使用电梯的用户的安全。

10

NOTE: Remote testing, control and intervention function can be used for troubleshooting, minimising mantrap occurrences and reducing rescue duration.

注意：远程测试、控制和干预功能可用于故障排除、最小化夹人事件的发生和减少救援时间。

6.  Cybersecurity Requirements网络安全要求.

- Scope范围

- This section specifies the cybersecurity requirements for the control system of the Remote Monitoring & Diagnostics Solution. This includes computers, operating systems, applications, and other programmable configurable components that form part of the security function of the system. Remote systems and services are excluded.

本节规定了远程监控和诊断解决方案控制系统的网络安全要求。这包括计算机、操作系统、应用程序和其他可编程可配置组件，这些组件构成了系统安全功能的一部分。远程系统和服务不在此范围之内。

- It is intended to cover Remote Monitoring & Diagnostics Solution installed in both new and existing lifts.

本节适用于安装在新建和现有电梯中的远程监控和诊断解决方案。

- General Requirements一般要求

- The fundamental requirement is to address cybersecurity threats in the Remote Monitoring & Diagnostics Solution lifecycle which includes product development, verification, operation, and maintenance. It focuses on the operational security function of the system that prioritizes protection of people, assets and information as well as the prevention of unauthorized use of monitor and/or control devices, processes.

基本要求是应对远程监控和诊断解决方案的生命周期中的网络安全威胁，包括产品开发、验证、运营和维护。重点是系统的操作安全功能，优先保护人员、资产和信息，并防止未经授权使用监控和/或控制设备、过程。

- IEC 62443-3-3:2013 and IEC 62443-4-2:2019 should be referenced when addressing cybersecurity threats for Remote Monitoring & Diagnostics Solution.

在处理远程监控和诊断解决方案的网络安全威胁时，应参考IEC 62443-3-3:2013和IEC 62443-4-2:2019。

6.3	Architecture and Design架构和设计

6.3.1	The architecture and design of the control system should meet IEC 62443 or equivalent.

控制系统的架构和设计应符合IEC 62443或等效标准。

6.4	Product Development Lifecycle产品开发生命周期

6.4.1	The product development lifecycle should be certified to IEC 62443-4-1 or equivalent.

产品开发生命周期应获得IEC 62443-4-1或等效认证。

6.5	Component and System Security组件和系统安全

- The components, specifically embedded devices, network components, host components and software applications, should be certified to IEC 62443-4-2 or equivalent.

各组件，特别是嵌入式设备、网络组件、主机组件和软件应用程序，应获得IEC 62443-4-2或等效认证

- The system security should be certified to IEC 62443-3-3 or equivalent.

系统安全性应获得IEC 62443-3-3或等效认证。

11

Annex A

附录A

(informative)

List of RM&D Indicators

以下是RM&D（远程监测与诊断）指标的列表

This section includes the following RM&D indicators, which should be used to evaluate whether the RM&D Solution is effective or not.

本节包括以下RM&D指标，这些指标应用于评估RM&D解决方案的有效性：

| S/N | Type of | Abbreviation |  |  |  |  |  |  |  |  | Formula |  |  |  |  |  |  |  | Units |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Indicators类型的指标包括 | 缩写 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Technical | TFPE |  |  | TFPE= |  | Total Number of Technical FaultsNOTE(1) |  |  |  |  |  |  |  |  |  |  |  | Technical Faults |  |
|  | Faults per |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Total Number of Equipment |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | per lift per month |  |
|  | Equipment设备的技术故障数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Faults per | FPE |  |  |  | FPE= |  |  |  |  | Total Number of FaultsNOTE(2) |  |  |  |  |  |  |  | Faults per lift per |  |
|  | Equipment设备故障数 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | month |  |
|  |  |  |  |  |  |  |  |  |  |  | Total Number of Equipment |  |  |  |  |  |  |  |  |  |
| 3 | First Time | FTTR |  | FTTRNOTE(3)= 1- Total Number of the Repeated Technical Faults |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % (per lift per |  |
|  | Fix Rate一次性修复率 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | month) |  |
|  |  |  |  |  |  |  |  |  |  |  | Total Number of Technical Faults |  |  |  |  |  |  |  |  |  |
| 4 | Mean Time | MTTR |  |  | MTTRNOTE(4)= |  |  |  |  | Total Downtime of Technical FaultsNOTE(5) |  |  |  |  |  |  |  |  | Hours/failure (per |  |
|  | To Repair平均修复时间 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | lift per month) |  |
|  |  |  |  |  |  |  |  |  |  |  | Total Number of Technical Faults |  |  |  |  |  |  |  |  |  |
| 5 | Average | UT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Monthly |  |  | Maximum Possible Running HoursNOTE(6)-Total Downtime of Technical Faults |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % (per lift per |  |
|  | Uptime |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | month) |  |
|  |  |  |  |  |  | Maximum Possible Running Hours |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 平均每月正常运行时间 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Diagnostics | DiA |  |  | Total Number of Intervention Cases marked as True (T)NOTE(7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Accuracy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % (per month) |  |
|  |  |  |  |  |  | Total Number of Intervention CasesNOTE(8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 诊断准确性 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | RM&D | DA |  |  |  | Total number of RM&D units that are online |  |  |  |  |  |  |  |  |  |  |  |  | % (per lift per |  |
|  | Device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Total number of RM&D units |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | month) |  |
|  | AvailabilityRM&D设备可用性 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE(1) A list of technical faults is given in Annex B.

附录B中列出了技术故障的清单

NOTE(2) Faults is sum of technical faults and non-technical faults. A list of non-technical faults is given in Annex B.

故障是技术故障和非技术故障的总和。非技术故障的清单列在附录B中。

NOTE(3) Technical faults that happen within the next 30 days after they have been rectified are to be considered for the calculations.
对于计算，应考虑在纠正后的30天内发生的技术故障。

12

NOTE (4) MTTR exclude the following: (1) major repair/overhaul that takes more than 1 day (refer to the list of exclusion cases below for more details); (2) waiting time for spare parts arrival; and (3) additional time needed to do hot-testing.

MTTR不包括以下内容：（1）需要超过1天的重大维修/大修（有关更多详细信息，请参阅下面的排除案例列表）；（2）等待备件到货的时间；以及（3）进行热测试所需的额外时间。

NOTE (5) Total downtime of technical faults is the sum of all time spent to rectify all technical faults in hours.

技术故障的总停机时间是指修复所有技术故障所花费的总时间，以小时为单位.

NOTE (6) Maximum possible running hours is the number of days in the month multiplied by 24 hours for each lift. 
最大可能运行小时数是每台电梯每个月的天数乘以24小时

NOTE (7) A True (T) Intervention Case is when the RM&D prediction matches diagnosis/faulty component on site.

真实（T）干预案例是指RM&D预测与现场诊断/故障部件相匹配的情况。

NOTE (8) Intervention cases are defined as cases prompted by RM&D Solution whereby a visit to lift by maintenance personnel is required.

干预案例被定义为由RM&D解决方案引发，需要维修人员访问电梯的情况。

|  | List of Exclusion Cases |
| --- | --- |
| S/N | Examples of MTTR Exclusion Cases (non-exhaustive) 以下是MTTR排除情况的示例（非详尽列表） |
| 1 | Hoisting motor replacement/repair曳引电机更换/修理 |
| 2 | Ropes replacement绳索更换 |
| 3 | Main/Diverting sheave replacement/repair 主/分流轮组更换/维修 |
| 4 | Major lift components, e.g. governor, safety gear重要的升降机部件，例如速度调节器、安全装置 |
| 5 | Total failure of Frequency Inverter频率变频器完全故障 |
| 6 | Water ingress situation水进入情况 |
| 7 | Building power failure大楼停电 |

13

Annex B

附录B

(informative)

This section includes examples of technical faults and non-technical faults.

本节包括技术故障和非技术故障的示例。

| S/N | Technical Faults技术故障 | Non-Technical Faults非技术故障 |
| --- | --- | --- |
| 1 | Motor电动机 | Noise噪音 |
| 2 | Machine brake [including Brake Monitoring Switch] 机械制动器[包括制动器监控开关] | Display Indicators/LCD显示指示器/LCD |
| 3 | Electrical Components [Switches/Contactors/Relays/PCBs] 电气元件[开关/接触器/继电器/电路板] | Faulty buttons [Car/Landing] [轿厢/层站]按钮故障 |
| 4 | Main Drive Unit/Frequency Inverter主驱动装置/频率变频器 | Card reader读卡器 |
| 5 | Landing Doors厅门 | External Element Blocking Doors [Object/Human] 外部元件阻挡门[物体/人类] |
| 6 | Car Doors轿厢门 | Car Interior [False Ceiling/Cladding] 汽车内饰 [假天花板/覆层] |
| 7 | Buffers缓冲器 | Fire Homing/Power Failure Mode消防自导/断电模式 |
| 8 | Speed Control System [Shaft/Motor Encoder] 速度控制系统 [轴/电机编码器] | Natural Disaster/Incident leading to component failure [Water |
|  |  | ingress] 导致组件故障的自然灾害/事件[水 入口] |
| 9 | Overspeed Governor & Governor Rope超速器和超速器绳 | CCTV Cameras监视器 |
| 10 | Levelling Accuracy水平精度 | Oil Pots Leakage油壶漏水 |
| 11 | ACOP/UCMP/Rope gripper ACOP/UCMP/绳夹 |  |
| 12 | Battery Failures [ARD/EBOPS] 电池故障 [ARD/EBOPS] |  |
| 13 | Suspension Ropes悬挂绳 |  |
| 14 | Worn out Bearings轴承磨损 |  |
| 15 | Load Measuring Devices负载测量装置 |  |
| 16 | Compensation Devices [Chain/Rope] 补偿装置[链/绳] |  |

14

Acknowledgements致谢

A Working Group on Remote Monitoring and Diagnostics was set up by BCA to prepare this document.

BCA成立了一个远程监控和诊断工作组，负责编制本文件

The organisations in which the experts of the Working Group are involved are:

工作组的专家所涉及的组织有:

- Building and Construction Authority (Chair)

- Land Transport Authority

- Housing Development Board

- Infocomm Media Development Authority

- Jurong Town Corporation

- Kone Pte Ltd

- Schindler Lifts (Singapore) Pte Ltd

- Otis Elevator Company (S) Pte Ltd

- Chevalier Singapore Holdings Pte Ltd

- Fujitec Singapore Corporation Ltd

- Mitsubishi Elevator Singapore Pte Ltd

- Hitachi Elevator Asia Pte Ltd

- E M Services Private Limited

- C&W Services Operations Pte Ltd

- Ken-Jo Industries Pte Ltd

- Surbana Jurong Private Limited

- TÜV SÜD PSB Pte Ltd

- Bosch

- Vision Elevator Components & Solutions Provider Pte Ltd

- Singapore Technologies Engineering Ltd

15